
[§5.2.6]  Translation Lookaside Buffers

The CPU generates virtual addresses, which correspond to locations in virtual memory.

In principle, the virtual addresses are translated to physical addresses using a page table.

	
[image: image6.png]Physically Indexed and Tagged
————
-

Virtually indexed but Physically Tagged

o=





	But this is too slow, so in practice, a translation lookaside buffer (TLB) is used.

It is like a special cache that is indexed by page number.

If there is a hit on a page number, then the address of the page in memory (called the page-frame address) is immediately obtained.


[image: image1.wmf]Page

table

Main

memory

0

1

2

3

1000

5000

8000

10000

Page #s

Phys.

addrs.

Therefore, the TLB and the cache must be accessed sequentially.

This adds an extra cycle in case of a hit.

(The page displacement is sometimes called the “page offset.”  But we will call it the displacement to avoid confusion with the block offset,” which we just call “offset.”)

How can we avoid wasting this time?

Let’s look at what happens when a memory address is accessed.

[image: image2.wmf] 

line select (mux)

 

word select (mux)

 

MD

R

 

row

 

dec

 

tag

 

(27 

bits)

 

31

 

5

 

4

 

3

 

2

 

0

 

tag 

(27)

 

index 

(2)

 

offset 

(3)

 

set (holds 2 blocks)

 

lines (8 bytes)

 

=?

 

=?

 

MAR

 


What are the steps in cache access?

We always need to read lines into the sense amplifiers and then select the word (cf. the direct-mapped cache diagram in Lecture 4).

Now, if we know the index before address translation takes place, we can perform steps 

 while address translation is occurring.

There is a tradeoff between speed and power efficiency.

· For power efficiency, which order should should steps 1 through 4 be performed in?  
· For maximum speed, which of steps 1 through 4 can be performed in parallel?  
Let’s take a look at address translation.

In this example, what is the page size?  
How much physical memory is there?  
Our goal is to allow the cache to be indexed before address translation completes.

In order to do that, we need to have the index field be entirely contained within the page displacement.  

So, if the displacement is d bits wide, the width of the index is j bits, and the offset is k bits, we must have j + k ( d.
Cache hit time reduces from two cycles to one!

… because the cache can now be indexed in parallel with TLB (although the tag match uses output from the TLB).

But there are some constraints...

•
Suppose our cache is direct mapped.  Then the index field just contains the line number.  So, (line number || block offset) must fit inside the page displacement.


What is the largest the cache can be?  
•
If we want to increase the size of the cache, what can we do?  

Options:

•
For new machines, select page size such that—

page size  (   EQ \f(cache size,associativity) 
•
If page size is fixed, select associativity so that—

associativity  (   EQ \f(cache size,page size) 
Example: MC88110

•
Page size = 4KB

•
I-cache, D-cache are both: 8KB, 2-way set-associative (4KB = 8KB / 2)

Example: VAX series

•
Page size = 512B

•
For a 16KB cache, need assoc. = (16KB / 512B) = 32-way set. assoc.!

The textbook gives these three alternatives for cache indexing and tagging.  Answer some questions about them.
What’s the main disadantage of physically indexed and tagged?  

What is the organization we have just been discussing (in the last diagram)?  

What is the main disadvantage of virtually indexed and tagged?  




  
Multilevel cache design

What are distinguishing features of the different cache levels of the four-level design (from 2013) illustrated on p. 135 of the textbook?

	
	Distinguish- ing feature
	Size
	Access time
	Implement’n techology

	L1 cache
	
	32KB–64KB
	2–3 cycles
	

	L2 cache
	
	256KB–1MB
	10–20 cycle
	

	L3 cache
	
	8–80 MB
	20–50 cycle
	

	L4 cache
	
	30–100 MB
	50–80 cycle
	

	Main mem.
	
	4–32 GB
	120–400 cy
	


What are some advantages of a centralized cache?  
What are some advantages of a banked structure?  
Inclusion in multilevel caches

Answer these questions about inclusion policies.
Which kind(s) of caches move a block from one level to the other?  

Which kind(s) of caches propagate up an eviction from the L2 to the L1?  
Which kind(s) of caches have to inform the L2 about a write to the L1?  
In an inclusive cache, can L2 associativity be greater than L1 associativity?  

Find and describe the typo in this diagram.  

[image: image3.png]@ Fundsmentals of parllel multics X @ Fundamentas of aralel Mulice X

< c 0

Apps [ Hub Spring 2023 .

Personal hub - Goo.

Fundamentals of Parallel Multicore Archi

+

& www-taylorfrancis-com.prox.ib.ncsu.edu/books/edit/10.1201/b20200.

2

* B emy

[] Courserrelated @ Messages [ Research basics

(175 of 495)

- o

X

[olE s I

[ Other boskmarks

~ 148 CHAPTER 5. INTRODUCTION TO MEMORY HIERARCHY ORGANIZATION
n Inclusive L2 Exclusive L2 NINEL2
After an L1 and L2 cache miss on block X and ¥ Z already in L2
L] B v v
el 1l J
v [
s
x 3
z z 3
Aferblock X s evited fromithe L1 cache:
- v v v
v v
s
X X %
3 3 H
Aerblock Y s evited fromithe L2 cache:
u i. i v
e L1zl
s
X 3 %
3 z
Y v
z z
e
s
X AR %
e z z
powersd
;2 Figure 5.8: llustration of various inclusion policies: inclusive, exclusive, and not-inclusive

and not-exclusive (NINE) L2 cache.





Replacement policies

LRU is a good strategy for cache replacement.

In a set-associative cache, LRU is reasonably cheap to implement.  Why?  
With the LRU algorithm, the lines can be arranged in an LRU stack,  in order of recency of reference.  Suppose a string of references is—

a  b  c  d  a  b  e  a  b  c  d  e

and there are 4 lines.  Then the LRU stacks after each reference are— 

a
b
c
d
a
b
e
a
b
c
d      e


a
b
c
d
a
b
e
a
b
c      d



a
b
c
d
a
b
e
a
b      c




a
b
c
d
d
d
e
a      b

*
*
*
*


*


*
*        *   

Notice that at each step:

•
The line that is referenced moves to the top of the LRU stack.

•
All lines below that line keep their same position.

•
All lines above that line move down by one position.

How many bits per set are required to keep track of LRU status in both of the implementations described in the text? 
· Matrix 
· Pseudo-LRU 
[image: image4.png]v - a x
@ Fundsmentals of parlll multics X @ Funcamentals of Paralie Mulic X 4

< C Y & www-taylorfrancis-com.proxlib.ncsu.edu/books/edit/10.1201/b20200.. & ¥ @ ® =W ¥ [l 3 I

Apps [ HubSpring 2023 -.. [ Personsl hub - Goo.. [ Courserrelted @ Messages || Ressarch basics 5 | [ ther bookmarks

Fundamentals of Parallel Multicore Archi (169 of 495)

142 CHAPTER 5. INTRODUCTION TO MEMORY HIERARCHY ORGANIZATION

Ateraceess to B At access to G Ateraceess to A
w0 "™ wayo ™ ey way0 e
wayo |0 |o|0]O way0 (oo |00 wayl | 0 0
wayt [4 [0 1] 1] et |1 ]o]o]1] vwan|o o
a0 o o]o] wae|1[1]0]1] wee|o o
waye [0 [0 |0]o] wae|ofofo|0]| wae|o o

Figure 5.6: lllustrating matrix implementation of the least recently used (LRU) replacement
policy.

least number of 1°s (way 2 has no 1°s), whereas the most recently used way has the most number of
1’s (way 3 has three 1's).

xx% ©





[image: image5.png]D = Y

@ Fundsmentals of parlll multics X @ Funcamentals of Paralie Mulic X 4

2 % @ ® =y [olE s I

C Y @& www-taylorfrancis-com.prox.lib.ncsu.edu/books/edit/10.1201/b20200.
» | [ Other bookmarks

<
Personalhub-Goo.. ] Courserelsied @ Messages [ Research basics

Apps [ Hub Spring 2023 .

Fundamentals of Parallel Multicore Archi (170 of 495)

; 5.2 BASIC ARCHITECTURES OF A CACHE 143
n Access Stream: Pseudo LRU Tree

B.C.ADE Access:
= 0=Lett, 1 = Rignt

Replacement:
0=Right, 1 = Leit

After accessto B After access to C After access to A
ale|c|o als|c|o als|c|o
Finging a block to repiace. After access to E,

At to make room for E Ereplaces B,
accesstoD Block B is selected Bits fiipped along the path

c|o Al E|lc|D

Aals|c|D Al B

Figure 5.7 lllustration of pseudo-LRU replacement on a 4-way set associative cache.

xx% ©





offset





index





tag





TLB





Cache





Main memory





physical address





Virtual address:





Page number





Displacement





miss





7





6





5





4





3





2





1





0





(Virtual) page number





Displacement





63





12





11





(Physical) page-frame #





Displacement





24





12





11





0





TLB





Tag





Offset








24





0





Index





TLB supplies


the physical page number portion





(Virtual) page number





Displacement





63





12





11





0





(Phys.) page-frame #





24





12





TLB





Tag





Offset





24





Indexex





12





Data array





Word select





Tag array





=?





0





11








© 2023 Edward F. Gehringer
CSC/ECE 506 Lecture Notes, Spring 2023


Lecture 5

Architecture of Parallel Computers



_1093813832.doc


Page







table







Main







memory







0







1







2







3







1000







5000







8000







10000







Page #s







Phys.







addrs.












_1092049634.doc


line select (mux)







word select (mux)







MDR







row







dec







tag







(27 bits)







31







5







4







3







2







0







tag (27)







index (2)







offset (3)







set (holds 2 blocks)







lines (8 bytes)







=?







=?







MAR












